ULTRA GRIPPER PRODUCT SPECIFICATIONS

Name of Product	      -           Ultra Grip Powder free Nitrile Gloves.

Name of Manufacturer       -           Innovative Gloves Co. Ltd. Hatyai, Thailand.

Base Material		       -          Nitrile.

Characteristics	       -          Non-Sterile, Ambidextrous.

Available Sizes	       -          Small, Medium, Large, X-Large, XX-Large, XXX-Large.

Surface		       -          Ultra Gripper.

Surface Treatment	       -          Polymer Coated.

Colour	                                 -          Orange.

Total Residue Content	       -          < 2.0 Milligrams per glove.

Physical Specifications:

| Size | Length(mm/inch) |  |  | Thickness(mm) Single Wall |  |  | Width(mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tip Smooth Surface | Tip Textured Surface |  | Palm Textured Surface | Cuff Smooth Surface |  |
| Small | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 85±5 |
| Medium | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 95±5 |
| Large | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 105±5 |
| X-Large | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 115±5 |
| XX-Large | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 125±5 |
| XXX-Large | 240±5 | 0.24±0.01 | 0.36±0.02 |  | 0.27±0.01 | 0.13±0.01 | 130±5 |

Tensile (D412 & EN 455-2)

Before Aging

| Tensile(MPA) | Elongation(%) | Force At Break(N) |
| --- | --- | --- |
| 22 Mpa Min. | 500 Min. | ≥ 6 N |

After Accelerated Aging

| Tensile(MPA) | Elongation(%) |
| --- | --- |
| 22 Mpa min. | 400 Min. |

Packaging

| Packaging (Normal) | 50±1,100±2 pieces in one Dispenser Box,10 Dispenser Box in one Carton. |
| --- | --- |
| Labeling on Dispenser (Normal) | Product Name, Size, Lot no. Quantity, Manufacturer’s address, Warning about Latex Allergy. |
| Labeling on Carton (Normal) | Product Name, Size, Lot no. Quantity, Country of Origin. |

Container Loadability (No MOQ in manufacturer owned brand in standard Packaging)

| 20 FCL=1135 Cartons. | 40 FCL=2280 Cartons. |
| --- | --- |

,

Certification